MODEL SB™ LATCHING TYPE PACKER PLUG AND MODEL SB™ LATCHING TYPE PACKER PLUG WITH SUPPORT MANDREL AND SURGE CHAMBER

- Product Family Nos. H66517, H66519, and H66520

DESCRIPTION/APPLICATION

The Model SB™ Latching Type Packer Plug is used to convert SC™, D™, and F™ style packers to temporary bridge plugs. When installed in the packer, the plug permits the operations such as pressure testing, perforating and washing above the packer without affecting the zone below the packer. The plug is equipped with an anchor latch and molded seals and is run on a Model SB™ Running Tool (Product Family No. H66531). The running tool slides over the control bar of the packer plug and is shear screwed in place. The running tool can be configured for either shear down or shear up release. Setting down approximately 3,000 lb (1,361 kg) latches the packer plug into the packer. Slacking-off or picking up 3,000 to 6,000 lb (1,361 to 2,722 kg) (depending on number of shear screws) releases the running tool. If debris settling on or around the packer plug is anticipated, gravel pack sand should be spotted over the plug to cover the control bar. The packer plug is retrieved with a Bowen® Overshot. The overshot should be equipped with a skirt to facilitate washing debris down to the top of the packer. After thoroughly washing around the plug, the overshot is latched onto the control bar. Straight pull of 6,000 to 8,000 lb (2,722 to 3,629 kg) shears the release mechanism of the plug and allows pressure equalization across the plug to allow it to be pulled out of the packer and retrieved. Alternately, the plug can be released with 15 turns right-hand rotation while holding a slight upstrain.